Lab Report:2

Name: Pranav Shankar

Roll No. :2023112011

Group No.: 8

Table No.:35

Experiment 1

Objective: Identification of logic levels with potentiometer

Electronic Component Used:

- Battery

- Hex Invertor (NOT gate)

- Multimeter

- Potentiometer

- Red and Green LED bulbs

- Wire

The Reference Circuit:

Procedure:

- First of all, we connected the vcc and ground of potentiometer.

- Then, we connected the multimeter with vcc and its other end with a LED bulb to ground, NOT GATE (Hex Invertor) and with the potentiometer parallel to each other.

- At last, we connected the other end of NOT GATE with another LED bulb which was later connected to ground.

Observation:

When the input voltage is low (0 ≤ VIL≤ 0.8), the output voltage is max=4.99V (2.4 ≤ VOH ≤ 5.0). And when input voltage is high, (2.0 ≤ VIH ≤ 5.0), the output voltage is low (0 ≤ VOL ≤ 0.4).

Low INPUT:

High INPUT:

Link for the Tinkercad simulation:

Link :

https://www.tinkercad.com/things/0Z6gTuCyIPJ-lab2exp1/editel?sharecode=mPulkGzfiz5qX4b-ZovOYdEndbLsllKsnBfeKPCwjCs

Experiment 2

Objective: Identification of various GATE

Electronic Component Used:

- Logic GATES (AND, OR, NAND, NOR, XOR, NOT)

- Battery

- LED bulb

- Wire

- Switches

The Reference Circuit:

AND						       OR

NAND						      NOR

XNOR						     NOT

Procedure:

For TTL 74LSxx(except NOT GATE) ICs

- First of all, we connected the vcc and ground of LOGIC GATE.

- Then, we connected two switches parallel to each other.

- Then, we connected the other two ends of switches with the LOGIC GATE input pin 1 and 2.

- From the output of LOGIC GATE pin 3, we connected the LED bulb and then to the ground.

For CMOS CD40xx ICs

- We did the same steps as for TTL 74LSxx ICs but instead for connecting the switches to pin 5 and 6.

- For the output we will collect it from pin 4.

Observation:

From the experiment, we observed the following;

For TTL 74LS04

| INPUT | OUTPUT |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

For TTL 74LS08

| A | B | OUTPUT |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

For TTL 74LS32

| A | B | OUTPUT |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

For TTL 74LS00

| A | B | OUTPUT |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

For TTL 74LS86

| A | B | OUTPUT |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

For CMOS CD4001

| A | B | OUTPUT |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

Conclusion:

From the experiment we can identify gate as following:

| GATE Series | GATE Name |
| --- | --- |
| TTL 74LS04 | NOT |
| TTL 74LS08 | AND |
| TTL 74LS32 | OR |
| TTL 74LS00 | NAND |
| TTL 74LS86 | XOR |
| CMOS CD4001 | NOR |

Tinkercad simulation Link and Lab Photo:

NOT GATE:

AND GATE:

Link:

https://www.tinkercad.com/things/97iw2p5Emmt-l2e2and/editel?sharecode=4gcg6CMp5Fp7yyIkI5qHXv4Q9Cg1iMBXUctpoITT-iA

OR GATE:

Link:

https://www.tinkercad.com/things/bcgf2PVsML2-l2e2or/editel?sharecode=60AbXFOJ_DW5FvFYTXsi2Nk7MGcInscr1Cc9-UvEysU

NAND GATE:

Link:

https://www.tinkercad.com/things/2qrDHRFmEOj-l2e2nand/editel?sharecode=1j6n_lDFXmYnER46zOWsKR-L_q2pynbId5BDLyq7zZ8

NOR GATE :

Link:

https://www.tinkercad.com/things/9YGtbPu5TPk-l2e2nor/editel?sharecode=huLQm_PkUrbm2utRIPvFdOh6zoMjvyLJkM7r533tINM

XOR GATE:

https://www.tinkercad.com/things/clzFd7BUbdY-l2e2xor/editel?sharecode=7G-o6CSkX0u5CkZX0SApS7OU00nLOikrwkVr5g7Yk-c

Experiment 3:

Objective: Checking De Morgan’s theorem using logic GATES

Electronic Component Used:

- Battery

- Logic GATES (AND, NAND, OR, NOR)

- LED bulbs

- Switches

- Wires

The Reference Circuit:

(A • B)’ = A’ + B’

(A + B)’ = A’ • B’

Procedure:

For (A • B)’ = A’ + B’

- First of all, we connected the vcc and ground of Logic GATES and two switches parallel to each other with battery.

- We connected the other end of two switches with two input pins of NAND Gate and the output pin with a LED bulb.

- We then connected parallelly from both the switches and connected to different inner circuit of NAND Gate such that in both the INPUT pin input come from same switch.

- Then, we will connect both output pins of NAND Gate with input pin of OR Gate.

- Then, we will connect it with LED bulb and then to ground.

For (A + B)’ = A’ • B’

- First of all, we connected the vcc and ground of Logic GATES and two switches parallel to each other with battery.

- We connected the other end of two switches with two input pins of NOR Gate and the output pin with a LED bulb.

- We then connected parallelly from both the switches and connected to different inner circuit of NAND Gate such that in both the INPUT pin input come from same switch.

- Then, we will connect both output pins of NAND Gate with input pin of AND Gate.

- Then, we will connect it with LED bulb and then to ground.

Observation:

We observed that both the LED bulbs glow in following ways:

For (A • B)’ = A’ + B’

| A | B | (A • B)’ | A’ + B’ |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 |

For (A + B)’ = A’ • B’

| A | B | (A + B)’ | A’ • B’ |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 |

Conclusion:

As both the LED bulbs glow together, we can conclude that De Morgan’s Law is true.

Tinkercad simulation and Lab photo:

In the above photo D1 and d7 are together

Link:

https://www.tinkercad.com/things/g0IvE3C3B3i-l2e3/editel?sharecode=jRRSpY2BdazUjpH0XwMYnXg1c5tlDTCjyBb_I9NP-R8

Experiment 4:

Objective: Make a binary full adder using logic GATES

Electronic Component Used:

- Battery

- Logic GATES (AND and XOR)

- Wire

- Switches

- LED bulbs

The Reference Circuit:

Procedure:

- First of all, we connected three switches parallel to each other from the battery.

- We connected the first two switches with input pins of XOR Gate and its output with another input pin of XOR Gate and the other input pin with third switch.

- We connected the output pin of second XOR Gate with bulb indicating SUM.

- We connected the first two switches parallelly with input pins of AND Gate.

- We connected the third switch and output pin of first XOR Gate with another AND Gate.

- Then, we connected the output pin of both the AND Gate as input of another XOR Gate and then to LED bulb to ground and this bulb will indicate CARRY.

Observation:

We observed the SUM and CARRY in following ways:

| A | B | C | SUM | CARRY |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Tinkercad simulation and Lab photo:

In the above photo D6 is SUM and D8 is CARRY.

Link: https://www.tinkercad.com/things/gzVVFdnUTm1-l2e4/editel?sharecode=S2mLU75YEWhru_Dsqm_cmnswpD31DiOiepIZx9IPM7I