LAB REPORT 4

Name:- Ayush Kumar Gupta

ROLL NO:-  2023114001

OBJECTIVE – Designing ALU using 8:1 Multiplexer.

ELCTRONIC COMPONENTS REQUIRED -

- Digital test kit.

- 8 input multiplexers(74LS151).

- 2 input multiplexer (74LS157).

- Quad two input exor gate(74LS86).

PROCEDURE:

- Test the ICs, LED Lights and Switches.

- Connect ICs with GND, Power and Enable (Enable was 0).

- Connect the 2 Input mux gate as mentioned in the reference circuit.

- Connect inputs to EXOR Gate as given in circuit.

- Connect all Output from Gates to input of two 8:1 Multiplexer.

- Connect Select line to Multiplexer.

- Check the Output while toggling different combination of Switches.

Truth Table:

| F2 F1 F0 | ALU FUNCTION | Y1 | Y2 |
| --- | --- | --- | --- |
| 000 | 0 | - | 0 |
| 001 | A+B | - | A+B |
| 010 | A.B | - | A.B |
| 011 | A EXOR B | - | A exor B |
| 100 | A PLUS B | CARRY | SUM |
| 101 | A MINUS B | BORROW | DIFF |
| 110 | A PLUS B PLUS C | CARRY | SUM |
| 111 | A MINUS B MINUS C | BORROW | DIFF |

REFERENCE

CIRCUIT:

LINK FOR TINKERCAD SIMULATION :

https://www.tinkercad.com/things/erhuHdWMkWC-shiny-allis-elzing/editel?sharecode=SnIbyrEy4qZKIaBpDDDHOy7Ls366MLz1s9kG3AjSlIQ

OBSERVATION IN TINKERCARD :

LAB:

OBSERVATION TABLE

1) 0

| F2 | F1 | F0 | A | B | C | Y1 | Y0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | X | X | X | 0 | 0 |

- A OR B

| F2 | F1 | F0 | A | B | C | Y1 | Y0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 | X | 0 | 0 |
| 0 | 0 | 1 | 0 | 1 | X | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | X | 0 | 1 |
| 0 | 0 | 1 | 1 | 1 | X | 0 | 1 |

- A AND B

| F2 | F1 | F0 | A | B | C | Y1 | Y0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 | X | 0 | 0 |
| 0 | 1 | 0 | 0 | 1 | X | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | X | 0 | 0 |
| 0 | 1 | 0 | 1 | 1 | X | 0 | 1 |

- A XOR B

| F2 | F1 | F0 | A | B | C | Y1 | Y0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 0 | 0 | X | 0 | 0 |
| 0 | 1 | 1 | 0 | 1 | X | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 | X | 0 | 1 |
| 0 | 1 | 1 | 1 | 1 | X | 0 | 0 |

- A+B

| F2 | F1 | F0 | A | B | C | Y1 | Y0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | X | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | X | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | X | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 | X | 1 | 0 |

- A-B

| F2 | F1 | F0 | A | B | C | Y1 | Y0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 0 | X | 0 | 0 |
| 1 | 0 | 1 | 0 | 1 | X | 1 | 1 |
| 1 | 0 | 1 | 1 | 0 | X | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 | X | 0 | 0 |

- A+B+C

| F2 | F1 | F0 | A | B | C | Y1 | Y0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |

- A-B-C

| F2 | F1 | F0 | A | B | C | Y1 | Y0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

CONCLUSION:

An ALU was designed and tested that could perform 8 arithmetic/ logical operations.